KMTNet Camera Electronics Upgrade (KMT-CEU)

Science Image MEF Interface Control Document

Version 2.0 Final Draft - Archon Raw Verified

Date: 2026-06-19

| 항목 | 정의 |
| --- | --- |
| Project | KMTNet Camera Electronics Upgrade (KMT-CEU) |
| Document scope | STA Archon raw MK/NT pair와 KMT-CEU science MEF 구조 정의 |
| Confirmed raw files | KMTN.20260116.000001.MK.fits, KMTN.20260116.000001.NT.fits |
| Official chip order | M, K, N, T (MKNT) |
| Science controllers | 2 STA Archon controllers for science image readout |
| Guide controller | 1 separate STA Archon controller; guide FITS is outside this document |

# 1. Purpose and Scope

본 문서는 KMT-CEU 시스템에서 STA Archon 컨트롤러로 획득한 실제 raw 영상 구조를 기준으로 science image MEF 포맷을 정의한다. 본 문서는 기존 KMTNet CCD 및 초점면을 유지하되, readout/video electronics와 wall board, CCD clock/timing configuration을 STA 기반으로 교체하는 KMT-CEU 시스템을 대상으로 한다.

원시 Archon raw pair: MK, NT

Science chip order: M, K, N, T

Raw-to-MEF 변환 시 필요한 primary header, image extension, geometry keyword, binary table 정의

벌지 mass production과 장기 재처리를 위한 metadata 및 version tracking 정의

# 2. Confirmed Archon Raw Files

실제 STA Archon 컨트롤러 관측 샘플로 확인된 raw file pair는 다음과 같다. MK 파일은 M/K chip data와 관측 metadata를 포함하며, NT 파일은 N/T chip data를 포함한다. 확인된 NT 샘플은 기본 FITS structural header만 포함하므로, MEF primary header는 MK 파일을 master header source로 사용해야 한다.

| Raw file | Role | Contains | Header role |
| --- | --- | --- | --- |
| KMTN.20260116.000001.MK.fits | Master raw file | M, K chip data | 관측 metadata source |
| KMTN.20260116.000001.NT.fits | Companion raw file | N, T chip data | Data source only in confirmed sample |

| Keyword / property | MK sample | NT sample | Implication |
| --- | --- | --- | --- |
| BITPIX | 16 | 16 | Unsigned short with BZERO |
| NAXIS1 | 19200 | 19200 | 16 x 1200 column raw tiles |
| NAXIS2 | 9400 | 9400 | 4616 + 168 + 4616 row structure |
| BZERO / BSCALE | 32768 / 1 | 32768 / 1 | Preserve uint16 scaling |
| OBSERVAT | SSO | not present | MK should be master header |
| FILTER | I | not present | MK should be master header |
| OBJECT | BLG01 | not present | MK should be master header |
| PROJID | BLG | not present | MK should be master header |
| IMAGETYP / OBSTYPE | OBJECT / OBJECT | not present | MK should be master header |
| DATE-OBS / TSHOPEN | 2026-01-16T03:09:50 / 03:09:50.28 | not present | MK should be master header |
| AZ / SECZ / HA / ST | -165.6 / 1.00 / +00:01:15 / 17:41:39 | not present | MK should be master header |

# 3. KMT-CEU System Definition

| Category | Definition |
| --- | --- |
| Reused components | Existing e2v CCD290-99 detectors, detector mounting plate, focal plane assembly, cryostat, vacuum system |
| Replaced components | Legacy readout electronics, legacy wall board, legacy timing electronics |
| New electronics | STA Archon controllers, STA differential wall boards, new signal readout/video-chain electronics |
| Timing configuration | CCD clock and timing configuration implemented through Archon timing script/configuration |
| Science CCDs | 4 chips: M, K, N, T |
| Readout architecture | 8 vertical strips per CCD, 2 readout ends per strip, 16 amplifiers per CCD, 64 amplifiers total |

본 문서에서 “signal readout/video-chain electronics”는 CCD output node 이후의 analog signal path와 digital conversion path를 의미한다. 여기에는 STA differential wall board, differential signal transfer, Archon video input, CDS/ADC path, video gain/noise setting이 포함된다.

“CCD clock and timing configuration”은 CCD charge transfer와 readout을 제어하는 clock sequence, parallel/serial clock timing, reset timing, flush/readout pattern, TOP/BOT readout synchronization, Archon timing script version을 의미한다. 이는 DATE-OBS, UT, MJD-OBS 같은 관측 시각 metadata와 구별된다.

# 4. Verified Archon Raw Geometry

실제 MK/NT raw sample 기준으로 확인된 raw geometry는 다음과 같다.

| Dimension | Verified value | Interpretation |
| --- | --- | --- |
| NAXIS1 | 19200 | 16 raw amp tiles x 1200 columns |
| Raw amp tile width | 1200 columns | 1152 active columns + 48 overscan columns |
| Active columns per tile | 1152 | Legacy strip width retained |
| X overscan per tile | 48 | Actual Archon raw overscan; legacy 32 px와 다름 |
| X prescan per tile | 0 | Actual Archon raw sample has no X prescan |
| NAXIS2 | 9400 | 4616 lower active + 168 middle Y overscan + 4616 upper active |
| Active rows per readout end | 4616 | 9232 / 2 |
| Middle Y overscan | 168 rows | Rows 4617:4784; ignore for active science image assembly |
| Full active CCD size | 9216 x 9232 | 8 strips x 1152 columns, 2 x 4616 rows |

Raw X layout per MK/NT file:
  19200 = 16 x 1200
  1200  = 1152 active + 48 overscan

Raw Y layout:
  rows 1:4616       = lower active half
  rows 4617:4784    = middle Y overscan, 168 rows
  rows 4785:9400    = upper active half

Raw overscan placement in X is tile-dependent. The current conversion logic treats amps 1-4 and 9-12 as data followed by right-side overscan, while amps 5-8 and 13-16 are treated as left-side overscan followed by data. This convention shall be encoded explicitly in DATASEC and BIASSEC rather than inferred downstream.

# 5. Official Science Chip Order and Raw Grouping

| Item | Final definition |
| --- | --- |
| Official chip order | M, K, N, T |
| Primary keyword | CHIPLIST = 'M,K,N,T' |
| Raw file grouping | MK -> M,K ; NT -> N,T |
| Recommended Stage-1 MEF order | M extensions, K extensions, N extensions, T extensions |
| Recommended Stage-2 CCD-level order | SCI_M, SCI_K, SCI_N, SCI_T |

| Chip | Raw file | Position in file | Global amplifier/layer IDs |
| --- | --- | --- | --- |
| M | MK | Left half of MK raw image | 1-16 |
| K | MK | Right half of MK raw image | 17-32 |
| N | NT | Left half of NT raw image | 33-48 |
| T | NT | Right half of NT raw image | 49-64 |

# 6. Detector Mosaic Geometry

Legacy KMTNet MEF extension headers and the current converter geometry define the following mosaic coordinate system. Detector coordinate (1,1) is the lower-left corner of the N chip region.

Detector mosaic layout:

  Y=10166:19397      M             K
                       | 460 px gap |
  Y=1:9232           N             T

  X=1:9216           X=9677:18892

| Chip | Mosaic position | DETSEC |
| --- | --- | --- |
| M | Upper left | [1:9216,10166:19397] |
| K | Upper right | [9677:18892,10166:19397] |
| N | Lower left | [1:9216,1:9232] |
| T | Lower right | [9677:18892,1:9232] |

| Quantity | Value |
| --- | --- |
| CCD width | 9216 pixels |
| CCD height | 9232 pixels |
| Column gap | 460 pixels |
| Row gap | 933 pixels |
| Mosaic width | 18892 pixels |
| Mosaic height | 19397 pixels |

# 7. Strip, End, and Amplifier Convention

Each CCD keeps the legacy 8 vertical-strip geometry. KMT-CEU reads each strip from two ends, resulting in 16 amplifiers per CCD.

| Local amp range | Readout end | Active raw Y range | CCD Y range | Recommended ENDID |
| --- | --- | --- | --- | --- |
| 1-8 | Upper / top half | 4785:9400 | 4617:9232 | TOP |
| 9-16 | Lower / bottom half | 1:4616 | 1:4616 | BOT |

| Chip | Strip numbering direction | IMFLIP convention | Notes |
| --- | --- | --- | --- |
| M | +X | None | M01 is leftmost strip |
| K | -X | -X,-Y | K01 is rightmost strip |
| N | -X | -X,-Y | N01 is rightmost strip |
| T | +X | None | T01 is leftmost strip |

Geometry must not be inferred from chip order or strip number alone. STRIPDIR, CCDSEC, AMPSEC, DETSEC, DATASEC, BIASSEC, READDIR, and IMFLIP are mandatory geometry descriptors.

# 8. MEF Output Products

Two related MEF products are recommended. Stage-1 is the low-level readout-preserving MEF for calibration/debugging and raw validation. Stage-2 is the optional CCD-level product for archive and mass-production processing. The current converter is closest to Stage-1.

| Product | Purpose | Structure |
| --- | --- | --- |
| Stage-1 64-layer MEF | Readout-preserving conversion, electronics debugging, amp-level calibration | PRIMARY + M01T/M01B...T08T/T08B image extensions + optional binary tables |
| Stage-2 CCD-level MEF | User/archive-friendly science image, mass-production input | PRIMARY + SCI_M + SCI_K + SCI_N + SCI_T + AMPINFO + XTALKINFO + VOLTINFO + TELEMETRY |

Recommended Stage-1 extension order:
  M01T ... M08T, M01B ... M08B
  K01T ... K08T, K01B ... K08B
  N01T ... N08T, N01B ... N08B
  T01T ... T08T, T01B ... T08B

Current code-compatible names may be M01...M16, K01...K16, N01...N16, T01...T16, but ENDID must be added to disambiguate top/bottom.

Recommended Stage-2 MEF layout:
  PRIMARY
  SCI_M
  SCI_K
  SCI_N
  SCI_T
  AMPINFO
  XTALKINFO
  VOLTINFO
  TELEMETRY

# 9. Primary Header Requirements

Primary header shall combine legacy KMTNet observing metadata, confirmed Archon raw metadata, KMT-CEU electronics metadata, and long-term version tracking keywords. For confirmed raw pairs, MK shall be used as the master metadata source.

| Keyword | Example / value | Requirement |
| --- | --- | --- |
| CHIPLIST | M,K,N,T | Mandatory |
| RAWGROUP | MKNT | Mandatory |
| MKFILE | KMTN.20260116.000001.MK.fits | Mandatory |
| NTFILE | KMTN.20260116.000001.NT.fits | Mandatory |
| NUMFILES | 2 | Mandatory |
| RAWNAX1 | 19200 | Mandatory |
| RAWNAX2 | 9400 | Mandatory |
| RAWXTILE | 1200 | Mandatory |
| AMPDATA | 1152 | Mandatory |
| OVERSCNX | 48 | Mandatory for CEU raw |
| PRESCANX | 0 | Mandatory for CEU raw |
| MIDOVSCY | 168 | Mandatory for CEU raw |
| AMPYDATA | 4616 | Mandatory |
| CAMNAME | KMT-CEU | Mandatory |
| CONTROLL | STA ARCHON | Mandatory |
| NCTRL | 2 | Mandatory for science FITS |
| WBTYPE | STA Differential Board | Mandatory |
| SIGELEC | STA_DIFF_VIDEO | Recommended |
| TIMCONF | CEU_TIM_v1.0 | Recommended |
| XTALKVER / BIASVER / CLKVER | version strings | Mandatory for reprocessing |
| REFVER / CATVER / PIPEVER | version strings or N/A | Mandatory for production products |

# 10. Stage-1 Image Extension Header Requirements

| Keyword | Definition |
| --- | --- |
| EXTNAME | Recommended: M01T, M01B, ...; code-compatible: M01...M16 |
| CHIPID | M/K/N/T |
| STRIPID | 1-8 |
| ENDID | TOP or BOT |
| AMPSEQ | 1-16 within chip |
| AMPNAME2 | im1-im64, MKNT order |
| CCDSEC | CCD coordinate of active amp region; must include TOP/BOT Y range |
| AMPSEC | Amplifier/readout coordinate; may differ in orientation |
| DATASEC | Local extension data region |
| BIASSEC | Local extension overscan region |
| PRESEC | No prescan: [1:0,...] or N/A |
| TRIMSEC | Usually same as DATASEC |
| DETSEC | Mosaic detector coordinates |
| STRIPDIR | +X or -X |
| IMFLIP | None or -X,-Y |
| READDIR | +Y for BOT, -Y for TOP, unless STA wiring defines otherwise |
| ATV/LTV/DTV and ATM/LTM/DTM | Maintain for ds9/IRAF-style geometry compatibility |

# 11. AMPINFO Binary Table

AMPINFO is the authoritative geometry and electronics map. Even if Stage-1 uses 64 image extensions, AMPINFO should be generated to prevent downstream software from deriving geometry from extension names alone.

| Column | Type / example | Meaning |
| --- | --- | --- |
| AMPID | 1-64 | Global amplifier ID in MKNT order |
| CHIPID | M/K/N/T | Science chip ID |
| STRIPID | 1-8 | Vertical strip ID |
| ENDID | TOP/BOT | Readout end |
| STRIPDIR | +X/-X | Strip numbering direction |
| AMPSEQ | 1-16 | Local amplifier number |
| AMPNAME | M01T | Human-readable amp name |
| CTRLID | 1/2 | Science Archon controller ID |
| MODULE | TBD | Archon module number |
| CHANNEL | TBD | Archon channel number |
| CCDSEC | [x1:x2,y1:y2] | CCD active region |
| AMPSEC | [x1:x2,y1:y2] | Amplifier/readout region |
| DETSEC | [x1:x2,y1:y2] | Mosaic coordinates |
| DATASEC | [x1:x2,y1:y2] | Local data section |
| BIASSEC | [x1:x2,y1:y2] | Local overscan section |
| IMFLIP | None or -X,-Y | Legacy orientation convention |
| READDIR | +Y/-Y | Readout direction |
| GAIN/RDNOISE | float | Amp-level electronics properties |
| SATLEVEL/LINMAX | ADU | Amp-level saturation/linearity |

# 12. Additional Binary Table Extensions

| Extension | Purpose | Minimum columns |
| --- | --- | --- |
| XTALKINFO | Crosstalk model | SOURCE_AMP, TARGET_AMP, XTALK_COEF, XTALK_ERR, XTALK_VER |
| VOLTINFO | Archon bias/clock voltages | VOLTNAME, SETPOINT, MEASURED, UNIT, STATUS |
| TELEMETRY | Observation-time controller status | CTRLID, FWVERSION, BOARDTEMP, READTIME, STATUS, ERRORFLAG |

# 13. Implications for the Current Converter

Retain MK/NT grouping and MK as mandatory master header source.

Use MKNT as the official chip and AMPID ordering: M=1-16, K=17-32, N=33-48, T=49-64.

Record RAWNAX1=19200, RAWNAX2=9400, RAWXTILE=1200, AMPDATA=1152, OVERSCNX=48, PRESCANX=0, MIDOVSCY=168, AMPYDATA=4616.

Correct CCDSEC Y ranges so amps 1-8 map to 4617:9232 and amps 9-16 map to 1:4616 in CCD coordinates.

Add ENDID keyword or T/B suffix to extension names; M01...M16 alone is ambiguous.

Preserve BZERO/BSCALE for unsigned 16-bit raw data.

Add CEU electronics keywords: CAMNAME, CAMVER, ELECSYS, CONTROLL, NCTRL, WBTYPE, SIGELEC, TIMCONF, CTRLVER, TIMVER, BIASVER, CLKVER, XTALKVER.

Generate AMPINFO even for 64-layer MEF output.

Handle missing NT explicitly; if missing, create zero placeholders without trying to open the NT FITS file.

Do not use legacy PRESCANX=27 and OVERSCNX=32 for CEU raw frames; those are pre-upgrade values.

# 14. Remaining Open Items

| Item | Status |
| --- | --- |
| Final Archon controller-module-channel to amplifier wiring | STA/KASI confirmation required |
| Actual gain/read-noise per amplifier | Lab calibration required |
| Final top/bottom read direction signs | Confirm from readout test and overscan orientation |
| XTALK matrix | Calibration data required |
| VOLTINFO actual measured telemetry names | Archon software interface confirmation required |
| Guide camera FITS ICD | Separate document required |
| Stage-1 vs Stage-2 production policy | Operational decision required |

# Appendix A. Recommended Minimal Primary Header Additions

CHIPLIST= 'M,K,N,T'           / Official science chip order
RAWGROUP= 'MKNT    '           / Raw Archon file grouping convention
MKFILE  = 'KMTN.20260116.000001.MK.fits'
NTFILE  = 'KMTN.20260116.000001.NT.fits'
NUMFILES=                    2
RAWNAX1 =                19200
RAWNAX2 =                 9400
RAWXTILE=                 1200
AMPDATA =                 1152
OVERSCNX=                   48
PRESCANX=                    0
MIDOVSCY=                  168
AMPYDATA=                 4616
CAMNAME = 'KMT-CEU '
CONTROLL= 'STA ARCHON'
NCTRL   =                    2
WBTYPE  = 'STA Differential Board'
SIGELEC = 'STA_DIFF_VIDEO'
TIMCONF = 'CEU_TIM_v1.0'

# Appendix B. Stage-1 Extension Naming Recommendation

| Chip | TOP extensions | BOT extensions | Global AMPID |
| --- | --- | --- | --- |
| M | M01T-M08T | M01B-M08B | 1-16 |
| K | K01T-K08T | K01B-K08B | 17-32 |
| N | N01T-N08T | N01B-N08B | 33-48 |
| T | T01T-T08T | T01B-T08B | 49-64 |