KMTNet Camera Electronics Upgrade (KMT-CEU)

Science Image MEF Interface Control Document

Archon Raw Verified Update — Version 3.0 Draft

Date: 2026-06-19
Project: KMTNet Camera Electronics Upgrade
Scope: Science Camera raw-to-MEF conversion and archive metadata

This version incorporates actual Archon MK/NT raw image structure and the current kmtn2mef 64-layer converter behavior.

# Revision History

| Version | Date | Major changes |
| --- | --- | --- |
| 1.0 | 2026-06-18 | Initial KMT-CEU MEF keyword and Archon metadata definition. |
| 2.0 | 2026-06-19 | Updated with actual Archon raw MK/NT grouping and MKNT chip order. |
| 3.0 | 2026-06-19 | Updated readout-orientation policy: CEU shall not apply legacy chip-dependent flip; TOP/BOT readout geometry is common to all chips. |

# 1. Purpose and Scope

본 문서는 KMTNet Camera Electronics Upgrade (KMT-CEU)의 Science Image MEF 포맷과 Archon raw-to-MEF 변환 규칙을 정의한다. 기존 CCD, 초점면, cryostat은 유지하되 전자부는 STA Archon controller와 STA differential wall board 기반으로 교체되는 것을 전제로 한다.

본 버전은 실제 Archon 관측 raw 파일 한 쌍(MK/NT)과 현재 개발 중인 kmtn2mef 64-layer 변환 코드에서 확인된 구조를 반영한다. 문서의 목적은 원시자료 보존, 장기 재처리, 벌지 mass production, crosstalk/bleeding 분석, 전자계 상태 추적을 위한 공통 인터페이스를 정의하는 것이다.

# 2. Confirmed System Configuration

| Item | Definition / Value | Status |
| --- | --- | --- |
| Detector model | e2v CCD290-99 | Confirmed from legacy and CEU context |
| Science CCDs | 4 chips: M, K, N, T | Official order updated to MKNT |
| CCD active size | 9216 × 9232 pixels | Legacy geometry retained |
| Legacy readout | 8 vertical strips per CCD, 8 amps/CCD | OSU electronics era |
| CEU readout | 8 vertical strips × 2 readout ends = 16 amps/CCD | Archon system |
| Total science amps | 64 | 4 CCD × 16 amps |
| Science controllers | 2 STA Archon controllers | MK and NT raw file groups |
| Guide controller | 1 STA Archon controller | Separate guide FITS ICD |

# 3. Official Chip Order and Raw File Grouping

실제 Archon raw 파일과 변환 코드 흐름을 기준으로 science chip order는 MKNT로 정의한다. 이는 기존 문서의 KMTN 순서보다 실제 raw grouping 및 변환 구현과 더 잘 일치한다.

Official science chip order: M, K, N, T
CHIPLIST = 'M,K,N,T'

Raw file grouping:
  MK file -> M, K
  NT file -> N, T

| Raw file tag | Contained chips | Role |
| --- | --- | --- |
| MK | M, K | Master header source and data source for M/K |
| NT | N, T | Data source for N/T; may have minimal header |

Operational rule: MK 파일은 master header source로 사용한다. NT 파일은 data source로만 취급해도 되도록 설계한다. NT 파일이 header-minimal인 경우에도 MK header를 사용하여 MEF primary header를 구성한다.

# 4. Verified Archon Raw Image Geometry

실제 Archon raw 영상에서 확인된 구조는 다음과 같다. 이 값은 KMT-CEU raw-to-MEF 변환의 기본 geometry로 사용한다.

| Keyword / Quantity | Value | Interpretation |
| --- | --- | --- |
| NAXIS1 | 19200 | 16 x-direction tiles per MK/NT raw file |
| NAXIS2 | 9400 | 4616 lower active + 168 middle Y overscan + 4616 upper active |
| BITPIX | 16 | Integer raw image |
| BZERO / BSCALE | 32768 / 1 | Unsigned 16-bit convention |
| X tile width | 1200 | 1152 active columns + 48 overscan columns |
| PRESCANX | 0 | No X prescan in Archon raw |
| OVERSCNX | 48 | X overscan columns per tile |
| MIDOVSCY | 168 | Middle Y overscan rows |
| AMPYDATA | 4616 | Active rows per TOP/BOT amplifier region |

X direction:
  19200 = 16 × 1200
  1200 = 1152 active + 48 overscan

Y direction:
  9400 = 4616 lower active + 168 middle overscan + 4616 upper active

# 5. Mosaic Geometry

Legacy KMTNet focal plane geometry is retained. Detector coordinates use the four-chip mosaic with column and row gaps.

Detector coordinates

Y=10166:19397      M                 K
                   | 460 px gap |
Y=1:9232           N                 T

X=1:9216           X=9677:18892

| Chip | Position | DETSEC |
| --- | --- | --- |
| M | Upper-left | [1:9216,10166:19397] |
| K | Upper-right | [9677:18892,10166:19397] |
| N | Lower-left | [1:9216,1:9232] |
| T | Lower-right | [9677:18892,1:9232] |

| Quantity | Value |
| --- | --- |
| CCD width | 9216 pixels |
| CCD height | 9232 pixels |
| Column gap | 460 pixels |
| Row gap | 933 pixels |
| Mosaic size | 18892 × 19397 pixels |

# 6. Legacy OSU Orientation vs. CEU Archon Orientation

기존 OSU camera electronics에서는 M/T와 K/N이 서로 다른 readout/storage orientation을 가졌다. Legacy extension header에서는 M/T가 IMFLIP=None, K/N이 IMFLIP=-X,-Y로 표현되었다. 그러나 KMT-CEU Archon 시스템에서는 모든 chip을 동일한 TOP/BOT dual-end readout scheme으로 읽으므로, 이 chip-dependent flip은 raw-to-MEF 변환 단계에서 적용하지 않는 것을 원칙으로 한다.

| System | M/T | K/N | Policy |
| --- | --- | --- | --- |
| Legacy OSU camera | Same orientation group | Opposite orientation group | Chip-dependent flip was required |
| KMT-CEU Archon | No chip-dependent flip | No chip-dependent flip | All chips use same TOP/BOT rule |

CEU policy:
  CHIPFLP = 'None' for M, K, N, T
  No chip-level X/Y flip is applied during Archon raw-to-MEF conversion.
  TOP/BOT readout end information is still mandatory.

따라서 CEU 문서와 코드에서는 legacy의 M/T vs K/N orientation 차이를 물려받지 않는다. 대신 amp 1–8과 amp 9–16이 각각 CCD의 TOP/BOT half를 담당한다는 universal geometry만 남긴다.

# 7. TOP/BOT Readout Geometry

| Amp range within chip | ENDID | CCD Y range | Raw Y range | Notes |
| --- | --- | --- | --- | --- |
| 1–8 | TOP | 4617:9232 | 4785:9400 | Upper active half; middle Y overscan ignored |
| 9–16 | BOT | 1:4616 | 1:4616 | Lower active half |

READDIR은 실제 STA clock/readout sequence 확인 후 최종 확정한다. 단, READDIR 규칙은 chip별로 달라지지 않고 TOP/BOT end에만 의존해야 한다.

| Keyword | Recommended value / rule | Meaning |
| --- | --- | --- |
| ENDID | TOP or BOT | Readout end within one vertical strip |
| READDIR | TBD, e.g. TOP=-Y, BOT=+Y | Physical readout direction; common rule for all chips |
| CHIPFLP | None | No chip-dependent flip in CEU |
| MIDOVSCY | 168 | Middle Y overscan rows ignored during MEF construction |

# 8. Amplifier and Layer Numbering

Layer numbering follows MKNT order and matches the current 64-layer converter concept.

| Chip | Layer / global amp range | Raw group | Position |
| --- | --- | --- | --- |
| M | 1–16 | MK | Upper-left |
| K | 17–32 | MK | Upper-right |
| N | 33–48 | NT | Lower-left |
| T | 49–64 | NT | Lower-right |

Within each chip, amp 1–8 correspond to TOP readout regions and amp 9–16 correspond to BOT readout regions. StripID is ((amp-1) mod 8)+1.

# 9. Section Keyword Definitions

The distinction between global CCD/mosaic coordinates and local extension coordinates is essential. CCDSEC and DETSEC must describe the physical CCD/mosaic region. DATASEC, BIASSEC, TRIMSEC, and PRESEC describe local coordinates within each output image extension.

| Keyword | Coordinate system | Required rule |
| --- | --- | --- |
| CCDSEC | CCD active coordinate | Must include TOP/BOT Y range: TOP=4617:9232, BOT=1:4616 |
| DETSEC | Full mosaic coordinate | Must include chip position and TOP/BOT Y range |
| DATASEC | Extension-local coordinate | Y=1:4616; X depends on overscan side |
| BIASSEC | Extension-local coordinate | 48-pixel X overscan; Y=1:4616 |
| TRIMSEC | Extension-local coordinate | Same as DATASEC for raw layer extension |
| PRESEC | Extension-local coordinate | [1:0,1:4616] because PRESCANX=0 |

| Amp group | Overscan side | DATASEC X | BIASSEC X |
| --- | --- | --- | --- |
| 1–4, 9–12 | Right | 1:1152 | 1153:1200 |
| 5–8, 13–16 | Left | 49:1200 | 1:48 |

# 10. Current 64-Layer MEF Output Structure

The current raw-to-MEF converter produces a 64-image-extension MEF, not yet the compact CCD-level SCI_M/SCI_K/SCI_N/SCI_T archive product. This 64-layer product is appropriate for debugging, commissioning, and electronics-level validation.

PRIMARY
M01 ... M16
K01 ... K16
N01 ... N16
T01 ... T16

| Header keyword | Example / rule | Description |
| --- | --- | --- |
| EXTNAME | M01 ... T16 | Layer extension name |
| CCDNAME | KMTNet CCD M | Chip name |
| AMPNAME | M01 | Local amp/layer name |
| AMPNAME2 | im1 ... im64 | Global layer number |
| REALDATA | T/F | Actual data or placeholder |
| CHIPFLP | None | Recommended new keyword |
| ENDID | TOP/BOT | Recommended new keyword |
| READDIR | TBD | Recommended new keyword |

# 11. Target Archive MEF Structure

For long-term archive and mass-production photometry, a compact CCD-level MEF remains recommended. The current 64-layer output can be converted to this product after electronics-level validation.

PRIMARY
SCI_M
SCI_K
SCI_N
SCI_T
AMPINFO
XTALKINFO
VOLTINFO
TELEMETRY

AMPINFO should become the authoritative geometry table, even if the current commissioning converter still stores most geometry directly in image extension headers.

# 12. Primary Header Additions for CEU

| Keyword | Example | Description |
| --- | --- | --- |
| CAMNAME | KMT-CEU | Camera electronics upgrade system |
| CAMVER | CEU-v1.0 | CEU system version |
| CHIPLIST | M,K,N,T | Official chip order |
| RAWGROUP | MKNT | Archon raw file grouping convention |
| MKCCDFILE | KMTN.<root>.MK.fits | MK raw source file |
| NTCCDFILE | KMTN.<root>.NT.fits | NT raw source file |
| NUMFILES | 2 | Number of raw Archon files used |
| RAWNAX1 | 19200 | Raw Archon image width |
| RAWNAX2 | 9400 | Raw Archon image height |
| RAWXTILE | 1200 | Raw tile width including overscan |
| AMPDATA | 1152 | Active columns per tile |
| OVERSCNX | 48 | X overscan columns per tile |
| PRESCANX | 0 | X prescan columns |
| MIDOVSCY | 168 | Middle Y overscan rows |
| AMPYDATA | 4616 | Active rows per TOP/BOT amp region |
| CHIPFLP | None | No chip-dependent flip in CEU |

# 13. Primary Operational Keywords to Preserve

The following operational keywords should be preserved from the existing KMTNet primary header or generated by the CEU acquisition system.

| Group | Keywords |
| --- | --- |
| FITS basic | SIMPLE, BITPIX, NAXIS, EXTEND, ORIGIN, DATE, OBJECT, BUNIT |
| Detector | DETECTOR, NAMPS, CCDXBIN, CCDYBIN, OVERSCNX, OVERSCNY, PRESCANX, PIXSCALE, PIXSIZE |
| Observatory | OBSERVAT, SITEID, TELESCOP, LATITUDE, LONGITUD, ELEVATIO, OBSERVER |
| Exposure | IMAGETYP, OBSTYPE, EXPTIME, DARKTIME, TSHOPEN, TSHSHUT, FILENAME, UNIQNAME, PROJID |
| TCS | TCSLINK, TCSARC, TCSQDATE, TCSUDATE, TIMESYS, DATE-OBS, RADECSYS, RA, DEC, EQUINOX, HA, ST, SECZ, ALT, AZ, TCSDRIVE, TELMOVE |
| Filter/Shutter | FSSTAT, FILTOP, FILNUM, FILTER, SHUTOP, SHUTTER |
| FSA environment | FSATEMP, FSAHUM, FSADEW, FSAALRM |
| Focus actuator | FASTAT, FAFOCUS, FATILTNS, FATILTEW, FAPOSS, FALIMS, FAPOSE, FALIME, FAPOSW, FALIMW |
| Dome | DSSTAT, DSUP, DSLW, DSSAF, DSAUTO, DSALT, DSAZ, DSTELALT, DSTELAZ, DALTERR, DAZERR |
| Mirror cover | MCSTAT, MCPOS |
| Thermal/cooling | CCDTEMP, DEWPRES, PT30N1, PT30N2, CHARCOAL, AIR_IN, AIR_OUT, GLYC_IN, GLYC_OUT |
| Version tracking | ELECSYS, CONTROLL, NCTRL, CTRLVER, TIMVER, XTALKVER, BIASVER, CLKVER, PIPEVER, REFVER, CATVER |

# 14. AMPINFO Extension Target Schema

AMPINFO is recommended for the target archive product. For the current 64-layer MEF, the same fields should either be present in each image extension header or be generated as AMPINFO in a later processing step.

| Column | Type | Meaning |
| --- | --- | --- |
| AMPID | Int | Global amp/layer ID, 1–64 in MKNT order |
| CHIPID | String | M, K, N, or T |
| AMPSEQ | Int | 1–16 within chip |
| STRIPID | Int | 1–8 within chip |
| ENDID | String | TOP or BOT |
| CHIPFLP | String | None for CEU |
| READDIR | String | Physical readout direction; TBD until clock validation |
| CTRLID | Int | Science Archon controller ID |
| MODULE | Int | Archon module ID |
| CHANNEL | Int | Archon channel number |
| CCDSEC | String | CCD active coordinate section |
| DETSEC | String | Detector mosaic coordinate section |
| DATASEC | String | Extension-local science data section |
| BIASSEC | String | Extension-local overscan section |
| PRESEC | String | No prescan: [1:0,1:4616] |
| TRIMSEC | String | Trimmed data section |
| GAIN | Float | Gain in e-/ADU |
| RDNOISE | Float | Read noise in e- |
| SATLEVEL | Int | Saturation level in ADU |
| LINMAX | Int | Linearity limit in ADU |

# 15. Converter Alignment Notes

The current kmtn2mef converter already aligns with several CEU decisions: chip list is MKNT, all chip-level flips are disabled, file grouping uses MK and NT, and middle Y overscan is detected and ignored. It also assigns layer IDs in MKNT order and constructs DETSEC Y ranges according to TOP/BOT and upper/lower chip positions.

However, the specification recommends one important correction: CCDSEC should use CCD-global TOP/BOT Y ranges. The current debug converter keeps CCDSEC Y as 1:stripeRows for all amps, while DETSEC already reflects TOP/BOT location. For consistency, CCDSEC should be updated to TOP=4617:9232 and BOT=1:4616.

| Item | Current behavior | Recommended final behavior |
| --- | --- | --- |
| Chip order | M,K,N,T | Keep |
| flipXY | False for all chips | Keep; add CHIPFLP=None keyword |
| Raw Y overscan | Middle block ignored | Keep |
| DETSEC Y | TOP/BOT reflected | Keep |
| CCDSEC Y | Currently local 1:4616 | Update to CCD-global TOP/BOT Y range |
| DATASEC/BIASSEC/TRIMSEC Y | Local 1:4616 | Keep |
| Missing NT handling | Needs robust branch | Use placeholder only if missing file is actually not opened |

# 16. Recommended Geometry Functions

def end_id(amp):
    return 'TOP' if 1 <= amp <= 8 else 'BOT'

def ccd_y_range(amp):
    return (4617, 9232) if end_id(amp) == 'TOP' else (1, 4616)

def local_y_range(amp):
    return (1, 4616)

def chip_flip(chip):
    return 'None'  # CEU Archon raw packing: no chip-dependent flip

def global_amp_id(chip, amp):
    base = {'M':0, 'K':16, 'N':32, 'T':48}
    return base[chip] + amp

# 17. Open Items

| Item | Status / Action |
| --- | --- |
| Physical READDIR sign | Confirm using STA timing and charge-transfer direction |
| Controller/module/channel mapping | Fill once Archon wiring table is finalized |
| Gain/read noise per amp | Measure after electronics commissioning |
| Crosstalk matrix | Generate XTALKINFO after bright-star/lab calibration |
| CCD-level archive MEF | Implement after 64-layer debug converter stabilizes |
| Guide FITS ICD | Separate document required |

# 18. Summary of Current Standard

Official chip order: M, K, N, T
Raw grouping: MK -> M,K ; NT -> N,T
Raw geometry: NAXIS1=19200, NAXIS2=9400
X tile: 1200 = 1152 active + 48 overscan
Y active: TOP=4617:9232, BOT=1:4616 in CCD coordinates
Middle Y overscan: 168 rows ignored
Chip-dependent flip: not applied in CEU
Layer numbering: M=1-16, K=17-32, N=33-48, T=49-64