KMTNet Camera Electronics Upgrade (KMT-CEU)

Science Image MEF Interface Control Document (ICD)

Final v1.1 | 2026-06-18

본 문서는 KMTNet Camera Electronics Upgrade (KMT-CEU)의 Science Image Multi-Extension FITS(MEF) 포맷, 주요 Primary Header 키워드, 실제 K/M/T/N CCD mosaic geometry, 16-output readout 구조, 그리고 AMPINFO/XTALKINFO/VOLTINFO/TELEMETRY binary table extension을 정의한다.

# Revision History

| Version | Date | Description |
| --- | --- | --- |
| Draft 0.1 | 2026-06-18 | Initial science MEF keyword draft |
| Draft 1.x | 2026-06-18 | KMT-CEU naming, STA Archon, STA differential wall board, guide camera separation |
| Draft 3.x | 2026-06-18 | AMPINFO, XTALKINFO, VOLTINFO, TELEMETRY binary table definitions |
| Draft 4.x | 2026-06-18 | Legacy K/M/T/N CCD geometry and strip ordering incorporated |
| Final v1.0 | 2026-06-18 | Primary keywords, operational telemetry, final MEF structure and geometry authority table consolidated |
| Final v1.1 | 2026-06-18 | Chip listing order standardized as K, M, T, N; electronics terminology clarified. |

# 1. Scope and Design Principles

KMT-CEU는 기존 KMTNet science camera의 CCD, focal plane, cryostat, vacuum system은 유지하면서, 기존 전자부와 wall board를 전면 교체하여 STA Archon controller와 STA differential wall board 기반으로 재구성하는 전자부 업그레이드 프로젝트이다.

Science CCD는 기존 e2v CCD290-99 4개를 유지한다.

Science focal plane geometry와 K/M/T/N chip identifier를 유지한다.

기존 8 amp/CCD readout에서 16 amp/CCD readout으로 확장한다.

한 CCD는 8개의 vertical strip으로 구성되며, KMT-CEU에서는 각 strip을 top/bottom 양 끝에서 읽는다.

Science FITS에는 science CCD를 제어하는 Archon controller 2대의 정보만 기록한다.

Guide camera는 별도의 FITS/ICD로 관리한다.

Raw science image에는 VAR/MASK image를 포함하지 않는다. VAR/MASK는 사용자 또는 pipeline 산출물로 처리한다.

AMPINFO table은 amplifier geometry와 electronics mapping의 authoritative source이다.

# 2. System Configuration

## 2.1 Reused and New Components

| Category | Item | Status |
| --- | --- | --- |
| Detector | e2v CCD290-99 science CCDs | Reused |
| Focal plane | Detector Mounting Plate (DMP) | Reused |
| Focal plane | Focal Plane Assembly (FPA) | Reused |
| Thermal/Vacuum | Cryostat and vacuum system | Reused |
| Electronics | STA Archon controllers | New |
| Electronics | STA differential wall boards | New |
| Electronics | Readout/video signal chain | New |
| Control | Archon timing scripts and readout configuration | New |

## 2.2 Detector and Controller Configuration

| Item | Value | Notes |
| --- | --- | --- |
| Science CCD count | 4 | K, M, T, N |
| CCD model | e2v CCD290-99 | Existing science detector |
| CCD image area | 9216 × 9232 pixels | Per science CCD |
| Legacy amplifiers | 8 amp/CCD | One amplifier per vertical strip |
| KMT-CEU amplifiers | 16 amp/CCD | Two readout ends per vertical strip |
| Total science amplifiers | 64 | 4 CCD × 16 amp |
| Science controllers | 2 STA Archon | Stored in science FITS header and TELEMETRY |
| Guide controller | 1 STA Archon | Guide FITS only; not included as science controller |
| Pixel size | 10.0 μm | Legacy value retained |
| Pixel scale | 0.400 arcsec/pixel | Unbinned pixel scale |

## 2.3 New Electronics Terminology

Chip listing convention: when the four science CCDs are listed as identifiers rather than shown in physical mosaic layout, the order shall be K, M, T, N. The physical mosaic layout remains M-K in the upper row and N-T in the lower row.

Readout/video signal chain means the CCD output signal acquisition path from the CCD output amplifiers through the STA differential wall board to the STA Archon video inputs, sampling electronics, analog-to-digital conversion, and digital readout path. In this document, “video” refers to CCD analog video signals, not a guide-camera video stream. This term includes video gain, offset/bias level, ADC behavior, correlated sampling configuration if applicable, cable/wall-board signal transmission, and noise/crosstalk behavior of the acquisition chain.

Archon timing scripts and readout configuration means the programmable clocking and sequencing recipe used by the Archon system to operate the CCDs. It includes parallel/serial clock timing, reset timing, charge transfer sequence, sampling windows, pixel time, line time, flushing pattern, exposure/readout sequencing, 64-amplifier readout mode, and the configuration versions that define these behaviors. This is tracked through TIMVER, CLKVER, BIASVER, READMODE, and related electronics metadata.

# 3. Science MEF Structure

KMT-CEU science raw data shall be stored as a Multi-Extension FITS file. The primary HDU stores global metadata. Four image extensions store CCD-level science images. Binary table extensions store amplifier geometry, crosstalk information, voltage configuration, and controller telemetry.

PRIMARY

SCI_K

SCI_M

SCI_T

SCI_N

AMPINFO

XTALKINFO

VOLTINFO

TELEMETRY

VAR/WEIGHT/MASK image extensions are intentionally excluded from the raw science MEF product. They may be generated later by the data reduction pipeline or by end users.

# 4. Legacy KMTNet Mosaic Geometry

기존 KMTNet geometry를 보존하는 것은 CEU 이후 reference image, catalog, WCS, DIA pipeline, mass-production light curve 재처리와 직접 관련된다.

## 4.1 Focal Plane Layout

Detector mosaic coordinates

Y = 19397

+-------------------------+-------------------------+

|            M            |            K            |

|     upper-left CCD      |     upper-right CCD     |

+-------------------------+-------------------------+

|            N            |            T            |

|     lower-left CCD      |     lower-right CCD     |

+-------------------------+-------------------------+

Y = 1

X = 1                   gap                   X = 18892

| CCD | Position | DETSEC | STRIPDIR | IMFLIP |
| --- | --- | --- | --- | --- |
| K | Upper right | [9677:18892,10166:19397] | -X | -X,-Y |
| M | Upper left | [1:9216,10166:19397] | +X | None |
| T | Lower right | [9677:18892,1:9232] | +X | None |
| N | Lower left | [1:9216,1:9232] | -X | -X,-Y |

| Parameter | Value | Description |
| --- | --- | --- |
| CCD width | 9216 px | 8 strips × 1152 px |
| CCD height | 9232 px | Full science height |
| Column gap | 460 px | Between left and right CCD columns |
| Row gap | 933 px | Between lower and upper CCD rows |
| Mosaic width | 18892 px | 9216 + 460 + 9216 |
| Mosaic height | 19397 px | 9232 + 933 + 9232 |
| Legacy strip width | 1152 px | Science pixels per vertical strip |
| Legacy raw strip width | 1211 px | 27 prescan + 1152 data + 32 overscan |

## 4.2 Legacy Stripe Geometry

기존 카메라에서는 각 CCD가 8개의 vertical stripe image extension으로 저장되었다. 예: M01 ... M08, K01 ... K08. 각 stripe의 raw image size와 section keyword는 다음과 같다.

| Keyword | Example | Description |
| --- | --- | --- |
| NAXIS1 | 1211 | Raw stripe width |
| NAXIS2 | 9232 | Raw stripe height |
| PRESEC | [1:27,1:9232] | Prescan section |
| DATASEC | [28:1179,1:9232] | Science data section in raw stripe |
| BIASSEC | [1180:1211,1:9232] | Bias overscan section |
| TRIMSEC | [28:1179,1:9232] | Trimmed science section |

## 4.3 Chip-dependent Stripe Ordering

| Chip | Legacy stripe direction | Examples | Requirement |
| --- | --- | --- | --- |
| K | K01 → K08 increases in -X | K01: [17741:18892], K08: [9677:10828] | STRIPDIR=-X |
| M | M01 → M08 increases in +X | M01: [1:1152], M08: [8065:9216] | STRIPDIR=+X |
| T | T01 → T08 increases in +X | T01: [9677:10828], T08: [17741:18892] | STRIPDIR=+X |
| N | N01 → N08 increases in -X | N01: [8065:9216], N08: [1:1152] | STRIPDIR=-X |

Geometry shall not be inferred from strip number alone. The combination of CHIPID, STRIPID, STRIPDIR, CCDSEC, AMPSEC, DETSEC, IMFLIP, and READDIR must be used.

# 5. KMT-CEU 16-output Readout Architecture

KMT-CEU retains the legacy eight-strip CCD geometry but uses the full 16-output capability of the e2v CCD290-99. Each vertical strip is read from the bottom and top ends.

Legacy camera:

1 CCD = 8 vertical strips × 1 readout end = 8 amp/CCD

KMT-CEU:

1 CCD = 8 vertical strips × 2 readout ends = 16 amp/CCD

4 CCD = 64 science amplifiers

## 5.1 Amplifier Naming

| Element | Example | Meaning |
| --- | --- | --- |
| CHIPID | M | CCD identifier: K, M, T, N |
| STRIPID | 01 | Legacy vertical strip number: 01 ... 08 |
| ENDID | BOT / TOP | Bottom or top readout end of a strip |
| AMPNAME | M01B / M01T | CEU amplifier name |
| STRIPDIR | +X / -X | Direction in detector coordinates in which STRIPID increases |

Final global AMPID numbering shall be determined after STA wiring definition. AMPID must be unique from 1 to 64, but geometry shall remain independent of AMPID order.

# 6. Primary Header Keyword Definition

Req: M=Mandatory, R=Recommended, O=Optional. Type/Unit values are intended for interface definition; exact FITS card formatting may be adjusted by the control software.

## 6.1 FITS Basic and File Identity

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| SIMPLE | M | Logical | T | FITS standard flag |
| BITPIX | M | Integer | 16 | Bits per pixel for raw science data |
| NAXIS | M | Integer | 0 | Primary HDU has no image array |
| EXTEND | M | Logical | T | MEF extensions are present |
| ORIGIN | M | String | KASI | FITS file originator |
| DATE | M | UTC string | 2028-04-15T12:34:56 | FITS file generation time |
| BUNIT | M | String | ADU | Data unit |
| FILENAME | R | String | kmtc.20280415.012345.fits | MEF filename |
| UNIQNAME | R | String | 280415.001 | Unique filename token |
| IRAF-TLM | O | UTC string | 2028-04-15T12:35:01 | Legacy last modification timestamp |

## 6.2 Observation Identification and Timing

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| OBJECT | M | String | BLG501 | Observed object or field name |
| FIELDID | M | String | BLG501 | KMTNet field identifier |
| PROJID | M | String | BULGE | Project or observing program ID |
| IMAGETYP | M | String | OBJECT | Image type |
| OBSTYPE | M | String | OBJECT | Observation type |
| TIMESYS | M | String | UTC | Time system |
| DATE-OBS | M | UTC string | 2028-04-15T12:34:56 | UTC start time of observation |
| UT | R | UTC string | 2028-04-15T12:34:56.123 | Shutter open time reported by master IC |
| MJD-OBS | R | Float / day | 61812.524260 | Modified Julian Date at start |
| JD | R | Float / day | 2461813.024260 | Julian Date at start |
| MIDJD | R | Float / day | 2461813.025000 | Mid-exposure JD or legacy MIDJD |
| MIDHJD | R | Float / day | 2461813.022410 | Mid-exposure heliocentric JD |
| LJD | O | Float / day | 2461813. | Legacy integer Julian date keyword |
| EXPTIME | M | Float / sec | 120.0 | Exposure time |
| DARKTIME | R | Float / sec | 0.0 | Cumulative dark time |
| TSHOPEN | R | UTC time | 12:34:56.12 | Shutter open time |
| TSHSHUT | R | UTC time | 12:36:56.12 | Shutter close time |

## 6.3 Observatory and Telescope

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| OBSERVAT | M | String | CTIO | Observatory site |
| SITEID | M | String | CTIO | Site identifier: CTIO/SAAO/SSO |
| TELESCOP | M | String | KMTNet 1.6m #1 | Telescope name |
| LATITUDE | M | deg | -30:10:01.84 | Site latitude |
| LONGITUD | M | deg | +70:48:14.39 | Site longitude, legacy sign convention retained if used |
| ELEVATIO | M | m | 2140 | Site elevation |
| OBSERVER | R | String | auto | Observer or observing agent |
| INSTRUME | M | String | KMTS | Instrument name |

## 6.4 TCS Link and Pointing

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| TCSLINK | R | String | Up | TCS communication link status |
| TCSARC | R | String | Enabled | TCS auto recovery mode status |
| TCSQDATE | R | UTC string | 2028-04-15T12:34:50.000 | Last TCS query time |
| TCSUDATE | R | UTC string | 2028-04-15T12:34:50.000 | Last TCS update time |
| RADECSYS | M | String | ICRS | Coordinate system |
| RA | M | hh:mm:ss | 17:54:32.10 | Telescope right ascension |
| DEC | M | dd:mm:ss | -29:15:12.3 | Telescope declination |
| EQUINOX | M | Float / yr | 2000.0 | Coordinate equinox |
| HA | R | hh:mm:ss | +01:12:34 | Hour angle at start |
| ST | R | hh:mm:ss | 17:42:11 | Local sidereal time at start |
| SECZ | M | Float | 1.15 | Secant of zenith distance / airmass |
| ALT | M | deg | 60.2 | Telescope altitude |
| AZ | M | deg | 210.4 | Telescope azimuth |
| TCSDRIVE | R | String | Enabled | Telescope drive status |
| TELMOVE | R | String | Idle | Telescope motion status |

## 6.5 Detector and Camera

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| CAMNAME | M | String | KMT-CEU | Camera system name |
| CAMVER | M | String | CEU-v1.0 | Camera/electronics upgrade version |
| DETECTOR | M | String | e2v CCD290-99 | Science detector model |
| DETTYPE | M | String | SCIENCE | Detector data product type |
| NCCD | M | Integer | 4 | Number of science CCDs |
| NAMPS | M | Integer | 64 | Total science amplifiers |
| AMPPCD | M | Integer | 16 | Amplifiers per CCD |
| NSTRIP | M | Integer | 8 | Vertical strips per CCD |
| NEND | M | Integer | 2 | Readout ends per strip |
| CHIPLIST | M | String | K,M,T,N | Science chip identifiers |
| DETSIZE | M | String | [1:18892,1:19397] | Detector mosaic size in pixels |
| COLGAP | M | Integer / px | 460 | Horizontal inter-CCD gap |
| ROWGAP | M | Integer / px | 933 | Vertical inter-CCD gap |
| CCDXBIN | M | Integer | 1 | CCD X binning |
| CCDYBIN | M | Integer | 1 | CCD Y binning |
| OVERSCNX | M | Integer / px | 32 | Horizontal overscan pixels per amplifier/readout segment |
| OVERSCNY | M | Integer / px | 0 | Vertical overscan pixels |
| PRESCANX | M | Integer / px | 27 | Horizontal prescan pixels per amplifier/readout segment |
| PIXSCALE | M | arcsec/px | 0.400 | Unbinned pixel scale |
| PIXSIZE | M | micron | 10.0 | Unbinned pixel size |
| READMODE | M | String | 64AMP | Science readout mode |
| READARCH | M | String | 8STRIP_2END | Readout architecture |

## 6.6 KMT-CEU Electronics and Version Tracking

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| ELECSYS | M | String | KMT-CEU | Electronics system identifier |
| CONTROLL | M | String | STA ARCHON | Controller type |
| NCTRL | M | Integer | 2 | Number of science Archon controllers |
| CTRL1ID | M | String | 00003FFF1A9902F6 | Science controller 1 ID |
| CTRL1SN | M | String | A24001 | Science controller 1 serial number |
| CTRL1FW | M | String | 1.0.1104 | Science controller 1 firmware version |
| CTRL1IP | R | String | 192.168.1.101 | Science controller 1 IP address |
| CTRL2ID | M | String | 00003FFF1A9911A2 | Science controller 2 ID |
| CTRL2SN | M | String | A24002 | Science controller 2 serial number |
| CTRL2FW | M | String | 1.0.1104 | Science controller 2 firmware version |
| CTRL2IP | R | String | 192.168.1.102 | Science controller 2 IP address |
| WBTYPE | M | String | STA Differential Board | Wall board type |
| CTRLVER | M | String | ARCHON-v1.0 | Controller configuration version |
| TIMVER | M | String | CEU_TIM-v1.0 | Timing script version |
| BIASVER | M | String | CEU_BIAS-v1.0 | Bias configuration version |
| CLKVER | M | String | CEU_CLK-v1.0 | Clock configuration version |
| XTALKVER | M | String | CEU_XTALK-v1.0 | Crosstalk model version |

## 6.7 Filter/Shutter and FSA Environment

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| AUXLINK | R | String | Up | AUX control link status |
| AUXARC | R | String | Enabled | AUX auto recovery mode |
| AUXQDATE | R | UTC string | 2028-04-15T12:34:50.000 | Last AUX query time |
| AUXUDATE | R | UTC string | 2028-04-15T12:34:50.000 | Last AUX update time |
| FSSTAT | M | String | STANDBY | Filter-Shutter subsystem status |
| FILTOP | M | String | STANDBY | Filter operational status |
| FILNUM | M | Integer | 1 | Filter selector position |
| FILTER | M | String | I | Filter name in the beam |
| SHUTOP | M | String | STANDBY | Shutter operational status |
| SHUTTER | M | String | OPEN | Shutter position |
| FSATEMP | M | deg C | 12.4 | Filter/Shutter Assembly internal temperature |
| FSAHUM | M | %RH | 8.2 | Filter/Shutter Assembly internal relative humidity |
| FSADEW | R | deg C | -8.4 | Filter/Shutter Assembly dew point |
| FSAALRM | R | String | NORMAL | FSA environmental alarm status |

## 6.8 Focus Actuator

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| FASTAT | M | String | STANDBY | Focus actuator subsystem status |
| FAFOCUS | M | mm | -0.717 | Focus position offset |
| FATILTNS | M | arcsec | -262.5 | Focus tilt NS offset angle |
| FATILTEW | M | arcsec | +51.0 | Focus tilt EW offset angle |
| FAPOSS | M | mm | +0.567 | South focus actuator position |
| FALIMS | R | Integer | 0 | South actuator limit status |
| FAPOSE | M | mm | -1.143 | East focus actuator position |
| FALIME | R | Integer | 0 | East actuator limit status |
| FAPOSW | M | mm | -1.575 | West focus actuator position |
| FALIMW | R | Integer | 0 | West actuator limit status |

## 6.9 Dome and Mirror Cover

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| DSSTAT | M | String | OPEN | Dome shutter status |
| DSUP | O | String/Float | OPEN | Upper dome shutter position/status |
| DSLW | O | String/Float | OPEN | Lower dome shutter position/status |
| DSSAF | R | String | NORMAL | Dome safety status |
| DSAUTO | R | String | ENABLED | Dome auto-synchronization status |
| DSALT | M | deg | 60.3 | Dome slit altitude |
| DSAZ | M | deg | 210.1 | Dome slit azimuth |
| DSTELALT | R | deg | 60.2 | Telescope altitude used by dome control |
| DSTELAZ | R | deg | 210.4 | Telescope azimuth used by dome control |
| DSTEL | O | deg | 60.2 | Legacy dome telescope altitude keyword |
| DALTERR | R | deg | +0.1 | Dome-telescope altitude difference |
| DAZERR | R | deg | -0.3 | Dome-telescope azimuth difference |
| MCSTAT | M | String | OPEN | Mirror cover status |
| MCPOS | M | % | 100 | Mirror cover position |

## 6.10 Thermal, Cooling, Vacuum, and Environment

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| CHSTAT | R | String | STANDBY | Chiller status |
| CHOP | O | String | ON | Chiller cooling on/off |
| CHSET | O | deg C | 18.0 | Chiller set point temperature |
| CHPROC | O | deg C | 5.0 | Chiller protection temperature |
| ENSTAT | R | String | STANDBY | Environmental control system status |
| ENFAN | R | String | ON | Environmental system fan status |
| ENS1 | R | deg C / %RH | 9.3 | Environmental sensor 1 |
| ENS2 | R | deg C / %RH | 8.9 | Environmental sensor 2 |
| ENS3 | R | deg C / %RH | 10.2 | Environmental sensor 3 |
| ENS4 | R | deg C / %RH | 26.0 | Environmental sensor 4 |
| ENS5 | R | deg C / %RH | 10.6 | Environmental sensor 5 |
| ENS6 | R | deg C / %RH | 11.3 | Environmental sensor 6 |
| ENS7 | R | deg C / %RH | 0.0 | Environmental sensor 7 |
| DEWPRES | R | Torr | 2.1E-6 | Dewar pressure |
| CCDTEMP | M | deg C | -111.41 | CCD temperature |
| PT30N1 | M | deg C | -158.42 | Cooler temperature sensor 1 |
| PT30N2 | M | deg C | -162.49 | Cooler temperature sensor 2 |
| CHARCOAL | M | deg C | -182.91 | Charcoal getter/cold stage temperature |
| AIR_IN | R | deg C | +25.35 | Cooling air inlet temperature |
| AIR_OUT | R | deg C | +21.54 | Cooling air outlet temperature |
| GLYC_IN | R | deg C | +18.32 | Glycol inlet temperature |
| GLYC_OUT | R | deg C | +19.67 | Glycol outlet temperature |
| WEATHER | R | Integer/String | 6 | Weather state code |
| MOONEFCT | R | Float | 0.0 | Moon effect or sky brightness indicator |

## 6.11 Processing and Reprocessing Metadata

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| OVERSCAN | R | Logical | F | Overscan corrected flag |
| BIASCOR | R | Logical | F | Bias corrected flag |
| FLATCOR | R | Logical | F | Flat corrected flag |
| DARKCOR | R | Logical | F | Dark corrected flag |
| XTALKCOR | R | Logical | F | Crosstalk correction applied flag |
| PIPELINE | R | String | KMT-CEU-RAW | Pipeline or data packaging software |
| PIPEVER | M | String | PIPE-v1.0 | Pipeline version |
| REFVER | M | String | CEU_BLG_REF-v1.0 | Reference image version for derived products |
| CATVER | M | String | CEU_BLG_CAT-v1.0 | Catalog version for derived products |
| QCFLAG | R | Integer | 0 | Overall quality flag |
| CHKIMG | O | String |  | Legacy check image flag |
| CHKIMG_C | O | String |  | Legacy check image comment/status |

# 7. Science CCD Image Extensions

Each science image extension is a CCD-level image extension. Amplifier-level raw/legacy stripe metadata are stored in AMPINFO rather than separate image extensions.

| Extension | CHIPID | CCD position | DETSEC | Notes |
| --- | --- | --- | --- | --- |
| SCI_K | K | Upper right | [9677:18892,10166:19397] | 16 amp, 8 strips × 2 ends |
| SCI_M | M | Upper left | [1:9216,10166:19397] | 16 amp, 8 strips × 2 ends |
| SCI_T | T | Lower right | [9677:18892,1:9232] | 16 amp, 8 strips × 2 ends |
| SCI_N | N | Lower left | [1:9216,1:9232] | 16 amp, 8 strips × 2 ends |

## 7.1 Common CCD Extension Keywords

| Keyword | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| XTENSION | M | String | IMAGE | Image extension |
| BITPIX | M | Integer | 16 | Array data type |
| NAXIS | M | Integer | 2 | Number of array dimensions |
| NAXIS1 | M | Integer / px | 9216 | CCD image width after assembly |
| NAXIS2 | M | Integer / px | 9232 | CCD image height after assembly |
| PCOUNT | M | Integer | 0 | Number of parameters |
| GCOUNT | M | Integer | 1 | Number of groups |
| EXTNAME | M | String | SCI_M | CCD extension name |
| CCDNAME | M | String | KMTNet CCD M | CCD name |
| CHIPID | M | String | M | Chip identifier |
| NAMPS | M | Integer | 16 | Amplifiers for this CCD |
| NSTRIP | M | Integer | 8 | Vertical strips for this CCD |
| NEND | M | Integer | 2 | Readout ends per strip |
| CCDSUM | M | String | 1 1 | On-chip binning factors |
| CCDSEC | M | String | [1:9216,1:9232] | CCD section |
| DATASEC | M | String | [1:9216,1:9232] | CCD-level science data section |
| DETSEC | M | String | [1:9216,10166:19397] | Mosaic detector coordinates |
| TRIMSEC | R | String | [1:9216,1:9232] | Trimmed CCD data section |
| GAIN | R | e-/ADU | 1.6 | Mean CCD gain; amp-level values in AMPINFO |
| RDNOISE | R | e- | 7.0 | Mean CCD read noise; amp-level values in AMPINFO |
| SATURATE | M | ADU | 62000 | Saturation level |
| BZERO | M | Integer | 32768 | Unsigned 16-bit integer offset |
| BUNIT | M | String | ADU | Data unit |
| FILTER | M | String | I | Filter name |
| PROJID | M | String | BULGE | Project ID copied from primary |

## 7.2 WCS and Transform Keywords

| Keyword | Req | Example | Description |
| --- | --- | --- | --- |
| CTYPE1 | M | RA---TAN | Coordinate type for axis 1 |
| CTYPE2 | M | DEC--TAN | Coordinate type for axis 2 |
| CRVAL1 | M | 143.4458333333 | Reference RA |
| CRVAL2 | M | 4.6520555556 | Reference Dec |
| CRPIX1 | M | 9445 | Reference pixel X for CCD-level image |
| CRPIX2 | M | -466 | Reference pixel Y for CCD-level image |
| CD1_1 | M | -0.00011122222222 | Coordinate transformation matrix |
| CD1_2 | M | 0.0 | Coordinate transformation matrix |
| CD2_1 | M | 0.0 | Coordinate transformation matrix |
| CD2_2 | M | 0.000111222222222 | Coordinate transformation matrix |
| WCSDIM | R | 2 | Coordinate system dimensionality |
| WAT0_001 | O | system=image | Legacy IRAF WCS keyword |
| WAT1_001 | O | wtype=tan axtype=ra | Legacy IRAF WCS keyword |
| WAT2_001 | O | wtype=tan axtype=dec | Legacy IRAF WCS keyword |

# 8. AMPINFO Binary Table Extension

AMPINFO is the authoritative geometry and electronics map. It replaces the legacy practice of storing each amplifier/stripe as an image extension header. Each row represents one CEU amplifier region. There shall be 64 rows for science data.

| Column | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| AMPID | M | Int16 | 37 | Unique global amplifier ID |
| CHIPID | M | String | T | CCD identifier: K, M, T, N |
| CCDNAME | M | String | KMTNet CCD T | CCD name |
| STRIPID | M | Int16 | 1 | Vertical strip number: 1..8 |
| ENDID | M | String | BOT | Readout end: BOT or TOP |
| STRIPDIR | M | String | +X | Direction in detector coordinates where STRIPID increases |
| AMPSEQ | M | Int16 | 1 | Amplifier sequence within CCD: 1..16 |
| AMPNAME | M | String | T01B | CEU amplifier name |
| AMPNAME2 | R | String | im25B | Legacy-compatible or control software channel name |
| OLD_EXTNAME | R | String | T01 | Legacy stripe extension corresponding to this strip |
| CTRLID | M | Int16 | 2 | Science Archon controller number |
| MODULE | M | Int16 | 4 | Archon module number |
| CHANNEL | M | Int16 | 1 | Archon channel number |
| ADCCHAN | R | Int16 | 1 | ADC channel if distinct from CHANNEL |
| CCDSEC | M | String | [1:1152,1:4616] | CCD coordinates of amplifier region |
| AMPSEC | M | String | [1:1152,1:4616] | Amplifier coordinate section |
| DETSEC | M | String | [9677:10828,1:4616] | Detector mosaic coordinates |
| DATASEC | M | String | [28:1179,1:4616] | Science data section in amp readout image, if applicable |
| PRESEC | M | String | [1:27,1:4616] | Prescan section |
| BIASSEC | M | String | [1180:1211,1:4616] | Bias overscan section |
| TRIMSEC | M | String | [28:1179,1:4616] | Trimmed science section |
| IMFLIP | M | String | None | Raw stripe flip convention inherited from legacy geometry |
| READDIR | M | String | +Y | Physical/assembled readout direction |
| ATV1 | R | Float | 0 | CCD to amplifier transform vector 1 |
| ATV2 | R | Float | 0 | CCD to amplifier transform vector 2 |
| ATM1_1 | R | Float | 1 | CCD to amplifier transform matrix |
| ATM1_2 | R | Float | 0 | CCD to amplifier transform matrix |
| ATM2_1 | R | Float | 0 | CCD to amplifier transform matrix |
| ATM2_2 | R | Float | 1 | CCD to amplifier transform matrix |
| DTV1 | M | Float | 9676 | CCD/stripe to detector mosaic transform vector 1 |
| DTV2 | M | Float | 1 | CCD/stripe to detector mosaic transform vector 2 |
| DTM1_1 | R | Float | 1 | Detector mosaic transform matrix |
| DTM1_2 | R | Float | 0 | Detector mosaic transform matrix |
| DTM2_1 | R | Float | 0 | Detector mosaic transform matrix |
| DTM2_2 | R | Float | 1 | Detector mosaic transform matrix |
| GAIN | M | Float / e-/ADU | 1.6 | Amplifier gain |
| RDNOISE | M | Float / e- | 7.0 | Amplifier read noise |
| SATLEVEL | M | Int32 / ADU | 62000 | Saturation level |
| LINMAX | M | Int32 / ADU | 58000 | Linearity limit |
| XTALKGROUP | R | String | C1A | Crosstalk grouping label |

## 8.1 AMPINFO Example Rows

| AMPNAME | CHIPID | STRIPID | ENDID | STRIPDIR | CCDSEC | DETSEC | IMFLIP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M01B | M | 1 | BOT | +X | [1:1152,1:4616] | [1:1152,10166:14781] | None |
| M01T | M | 1 | TOP | +X | [1:1152,4617:9232] | [1:1152,14782:19397] | None |
| K01B | K | 1 | BOT | -X | [8065:9216,1:4616] | [17741:18892,10166:14781] | -X,-Y |
| K01T | K | 1 | TOP | -X | [8065:9216,4617:9232] | [17741:18892,14782:19397] | -X,-Y |
| N08B | N | 8 | BOT | -X | [1:1152,1:4616] | [1:1152,1:4616] | -X,-Y |
| T08T | T | 8 | TOP | +X | [8065:9216,4617:9232] | [17741:18892,4617:9232] | None |

The exact TOP/BOT DATASEC, BIASSEC, AMPSEC, and READDIR definitions shall be finalized after STA 16-output readout layout confirmation. The above rows illustrate the intended schema and geometry convention.

# 9. XTALKINFO Binary Table Extension

XTALKINFO stores the 64-amplifier crosstalk calibration. Full dense 64×64 matrices or sparse coefficient rows may be used. Primary header stores only XTALKVER; the table stores the model contents or references.

| Column | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| SOURCE_AMP | M | Int16 | 12 | Source amplifier ID |
| TARGET_AMP | M | Int16 | 13 | Target amplifier ID |
| XTALK_COEF | M | Float64 | 2.3E-5 | Crosstalk coefficient |
| XTALK_ERR | R | Float64 | 1.1E-6 | Coefficient uncertainty |
| XTALK_VER | M | String | CEU_XTALK-v1.0 | Crosstalk model version |
| MEASDATE | R | UTC string | 2028-04-15 | Measurement/calibration date |
| VALID | R | Logical | T | Coefficient validity flag |

# 10. VOLTINFO Binary Table Extension

VOLTINFO stores Archon bias and clock voltage configuration. It is intended for reproducibility and long-term electronics diagnostics.

| Column | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| CTRLID | M | Int16 | 1 | Science controller number |
| VOLTNAME | M | String | VOD | Voltage or clock rail name |
| SETPOINT | M | Float32 | 27.50 | Nominal set value |
| MEASURED | R | Float32 | 27.48 | Measured value |
| UNIT | M | String | Volt | Unit |
| STATUS | R | String | OK | Voltage status |
| BIASVER | M | String | CEU_BIAS-v1.0 | Bias configuration version |
| CLKVER | M | String | CEU_CLK-v1.0 | Clock configuration version |

# 11. TELEMETRY Binary Table Extension

TELEMETRY stores observation-time science Archon controller status. Guide controller telemetry belongs to the guide FITS product, not the science FITS product.

| Column | Req | Type / Unit | Example | Description |
| --- | --- | --- | --- | --- |
| CTRLID | M | Int16 | 1 | Science controller number |
| CTRLIDSTR | R | String | 00003FFF1A9902F6 | Controller ID string |
| FWVER | M | String | 1.0.1104 | Firmware version |
| BOARDTEMP | R | deg C | 36.5 | Controller or board temperature |
| READTIME | M | sec | 18.2 | Readout time |
| STATUS | M | String | OK | Controller status |
| ERRORFLAG | M | Int16 | 0 | Error flag |
| CMDERR | R | Logical | F | Command error flag |
| CMDTOUT | R | Logical | F | Command timeout flag |
| NRETRY | R | Int16 | 0 | Communication retry count |
| FETCHOK | R | Logical | T | Image fetch success flag |

# 12. Data Volume Estimate

| Product | Assumption | Approximate size |
| --- | --- | --- |
| Science image data only | 4 CCD × 9216 × 9232 px × uint16 | 0.63 GiB |
| AMPINFO + XTALKINFO + VOLTINFO + TELEMETRY | Binary tables, 64 amp metadata and calibration values | < few MB |
| Raw science MEF total | SCI_K/M/T/N + binary tables | about 0.65 GiB/exposure |
| Excluded from raw product | VAR/MASK/WEIGHT/DIA image products | Generated by user or pipeline |

# 13. Mandatory Long-Term Reprocessing Keywords

The following keywords are mandatory because they preserve the reproducibility of future bulge mass-production photometry and other long-term reprocessing campaigns.

CAMVER

ELECSYS

CTRLVER

TIMVER

XTALKVER

BIASVER

CLKVER

PIPEVER

REFVER

CATVER

# 14. Open Items Before Implementation

Exact STA Archon controller/module/channel to amplifier wiring map.

Final global AMPID numbering convention.

Final TOP/BOT DATASEC, PRESEC, BIASSEC, TRIMSEC definitions for 16-output readout.

Final AMPSEC and READDIR conventions after physical readout direction confirmation.

Crosstalk calibration procedure and XTALKINFO dense/sparse storage choice.

Final Archon telemetry set available from the control software.

Guide camera FITS ICD and science-guide OBSID/GUIDEID linking convention.

Whether CCD-level image extensions are stored already assembled by the acquisition software or by a pre-ingest raw packager; the AMPINFO geometry authority table must remain valid in either case.

# Appendix A. Recommended Minimal MEF Header Example

SIMPLE  =                    T

BITPIX  =                   16

NAXIS   =                    0

EXTEND  =                    T

ORIGIN  = 'KASI'

DATE    = '2028-04-15T12:34:56'

BUNIT   = 'ADU'

CAMNAME = 'KMT-CEU'

CAMVER  = 'CEU-v1.0'

DETECTOR= 'e2v CCD290-99'

DETTYPE = 'SCIENCE'

NCCD    =                    4

NAMPS   =                   64

AMPPCD  =                   16

NSTRIP  =                    8

NEND    =                    2

CHIPLIST= 'K,M,T,N'

CONTROLL= 'STA ARCHON'

NCTRL   =                    2

WBTYPE  = 'STA Differential Board'

ELECSYS = 'KMT-CEU'

TIMVER  = 'CEU_TIM-v1.0'

XTALKVER= 'CEU_XTALK-v1.0'

BIASVER = 'CEU_BIAS-v1.0'

CLKVER  = 'CEU_CLK-v1.0'

PIPEVER = 'PIPE-v1.0'

REFVER  = 'CEU_BLG_REF-v1.0'

CATVER  = 'CEU_BLG_CAT-v1.0'